Chapter 5

Question 1:

Which of the following statements about wired and wireless LANs is correct?

Responses:

1) Both operate in the lower two layers of the TCP/IP protocol suite.

2) Wired LANs operate in the lower two layers and wireless LANs operate in the lower three layers of the TCP/IP protocol suite.

3) Wired LANs operate in the lower three layers and wireless LANs operate in the lower two layers of the TCP/IP protocol suite.

4) Both operate in the lower three layers of the TCP/IP protocol suite.

Answer:

1) Correct Answer

Question 2:

IEEE has defined the specifications for a wireless LAN, called _______, which covers the physical and data link layers.

Responses:

1) IEEE 802.3

2) IEEE 802.5

3) IEEE 802.11

4) IEEE 802.2

Answer:

3) Correct Answer

Question 3:

The CSMA/CD algorithm does not work in wireless LAN because

Responses:

1) wireless host does not have enough power to work in s duplex mode.

2) of the hidden station problem.

3) signal fading could prevent a station at one end from hearing a collision at the other end.

4) All of the choices are correct.

Answer:

4) Correct Answer

Question 4:

In IEEE 802.11, a ___ is made of stationary or mobile wireless stations and an optional central base station, known as the access point (AP).

Responses:

1) ESS

2) BSS

3) CSS

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 5:

In IEEE 802.11, a BSS without an AP is called _________.

Responses:

1) an ad hoc architecture

2) an infrastructure network

3) either an ad hoc architecture or an infrastructure network

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 6:

In IEEE 802.11, a BSS with an AP is sometimes referred to as ____________.

Responses:

1) an ad hoc architecture

2) an infrastructure network

3) either an ad hoc architecture or an infrastructure network

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 7:

In IEEE 802.11, communication between two stations in two different BSSs usually occurs via two ________.

Responses:

1) BSSs

2) ESSs

3) APs

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 8:

In IEEE 802.11, a station with ________ mobility is either stationary (not moving) or moving only inside a BSS.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 9:

In IEEE 802.11, a station with ________mobility can move from one BSS to another, but the movement is confined inside one ESS.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 10:

In IEEE 802.11, a station with ________ mobility can move from one ESS to another.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 11:

In IEEE 802.11, distributed coordination function (DCF) uses _______ as the access method.

Responses:

1) CSMA/CA

2) CSMA/CD

3) ALOHA

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 12:

In IEEE 802.11, when a frame is going from one station in a BSS to another station in the same BSS, the address flag is _____

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

1) Correct Answer

Question 13:

In IEEE 802.11, when a frame is coming from an AP and going to a station, the address flag is _______.

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

2) Correct Answer

Question 14:

In IEEE 802.11, when a frame is going from a station to an AP, the address flag is _____.

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

3) Correct Answer

Question 15:

In IEEE 802.11, when a frame is going from one AP to another AP in a wireless distribution system, the address flag is _____

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

4) Correct Answer

Question 16:

In IEEE 802.11, the access method used in the PCF sublayer is ______.

Responses:

1) contention

2) controlled

3) polling

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 17:

In IEEE 802.11, the ______ is a time period used for collision avoidance.

Responses:

1) NAV

2) BSS

3) ESS

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 18:

In IEEE 802.11, the addressing mechanism can include up to ______addresses.

Responses:

1) four

2) five

3) six

4) None of the choices are Correct.

Answer:

1) Correct Answer

Question 19:

The original IEEE 802.11 uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

4) Correct Answer

Question 20:

The IEEE 802.11a, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

3) Correct Answer

Question 21:

The IEEE 802.11b, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

2) Correct Answer

Question 22:

The IEEE 802.11g, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

3) Correct Answer

Question 23:

The IEEE 802.11 FHSS uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

2) Correct Answer

Question 24:

The IEEE 802.11 or IEEE 802.11b DSSS uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

3) Correct Answer

Question 25:

The IEEE 802.11a, IEEE 802.11g, or IEEE 802.11n OFDM uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

4) Correct Answer

Question 26:

Bluetooth is a _______ technology that connects devices (called gadgets) in a small area.

Responses:

1) wired LAN

2) wireless LAN

3) VLAN

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 27:

In Bluetooth, multiple ________ form a network called a _________.

Responses:

1) scatternet; piconets

2) piconets: scatternet

3) piconets: bluenet

4) bluenet; scatternet

Answer:

2) Correct Answer

Question 28:

A Bluetooth network consists of _____ primary device(s) and up to ____ secondary devices.

Responses:

1) one; five

2) five; three

3) two; six

4) one; seven

Answer:

4) Correct Answer

Question 29:

In Bluetooth, the current data rate is ____Mbps

Responses:

1) 2

2) 5

3) 11

4) None of the choices are correct.

Answer:

4) Correct Answer

Question 30:

The access method in Bluetooth is ________.

Responses:

1) FDMA

2) TDD-TDMA

3) CDMA

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 31:

In Bluetooth, the _____ link is used when data integrity is more important than avoiding latency.

Responses:

1) SCO

2) ACL

3) ACO

4) SCL

Answer:

2) Correct Answer

Question 32:

Bluetooth uses ______ in the physical layer to avoid interference from other devices or other networks.

Responses:

1) DSSS

2) FHSS

3) FDMA

4) None of the choices are correct.

Answer:

2) Correct Answer