Chapter 4

Question 1:

Given the Ethernet address 01011010 00010001 01010101 00011000 10101010 00001111 in binary, what is the address in hexadecimal notation?

Responses:

1) 5A:88:AA:18:55:F0

2) 5A:81:BA:81:AA:0F

3) 5A:18:5A:18:55:0F

4) 5A:11:55:18:AA:0F

Answer:

4) Correct Answer

Question 2:

If an Ethernet destination address is 07:01:02:03:04:05, then this is an ______ address.

Responses:

1) unicast

2) multicast

3) broadcast

4) either multicast or broadcast

Answer:

2) Correct Answer

Question 3:

If an Ethernet destination address is 08:07:06:05:44:33, then this is an ______ address.

Responses:

1) unicast

2) multicast

3) broadcast

4) either multicast or broadcast

Answer:

1) Correct Answer

Question 4:

Which of the following could not be an Ethernet multicast destination address?

Responses:

1) 43:7B:6C:DE:10:00

2) 44:AA:C1:23:45:32

3) 46:56:21:1A:DE:F4

4) 48:32:21:21:4D:34

Answer:

1) Correct Answer

Question 5:

Which of the following could not be an Ethernet unicast destination address?

Responses:

1) B7:7B:6C:DE:10:00

2) 7B:AA:C1:23:45:32

3) 7C:56:21:1A:DE:F4

4) 83:32:21:21:4D:34

Answer:

3) Correct Answer

Question 6:

The _______ layer of Ethernet consists of the LLC sublayer and the MAC sublayer.

Responses:

1) data link

2) physical

3) network

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 7:

The _____ sublayer is responsible for the operation of the CSMA/CD access method and framing.

Responses:

1) LLC

2) MII

3) MAC

4) None of the choices are correct,

Answer:

3) Correct Answer

Question 8:

Each station on an Ethernet network has a unique _______ address imprinted on its network interface card (NIC).

Responses:

1) 16-bit

2) 32-bit

3) 64-bit

4) None of the choices are correct.

Answer:

4) Correct Answer

Question 9:

The minimum frame length for Ethernet is _______bytes.

Responses:

1) 32

2) 80

3) 128

4) None of the choices are correct

Answer:

4) Correct Answer

Question 10:

Fast Ethernet has a data rate of ________Mbps.

Responses:

1) 10

2) 100

3) 1000

4) 10,000

Answer:

2) Correct Answer

Question 11:

In _________, auto-negotiation allows two devices to negotiate the mode or data rate of operation.

Responses:

1) Standard

2) Fast Ethernet

3) Gigabit Ethernet

4) Ten-Gigabit Ethernet

Answer:

2) Correct Answer

Question 12:

Gigabit Ethernet has a data rate of ________Mbps.

Responses:

1) 10

2) 100

3) 1000

4) 10,000

Answer:

3) Correct Answer

Question 13:

In Ethernet addressing, if the least significant bit of the first byte is 0, the address is _________.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 14:

In Ethernet addressing, if the least significant bit of the first byte is 1, the address is _________.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 15:

In Ethernet addressing, if all the bits are 1s, the address is _________.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 16:

In the Ethernet, the _______field is actually added at the physical layer and is not (formally) part of the frame.

Responses:

1) CRC

2) preamble

3) address

4) SFD

Answer:

2) Correct Answer

Question 17:

In the Ethernet frame, the _______ field contains error detection information.

Responses:

1) CRC

2) preamble

3) address

4) SFD

Answer:

1) Correct Answer

Question 18:

The largest portion of the bandwidth for ADSL carries _______.

Responses:

1) voice communication

2) upstream data

3) downstream data

4) control data

Answer:

3) Correct Answer

Question 19:

Another name for the cable TV office is the _______.

Responses:

1) splitter

2) fiber node

3) combiner

4) head end

Answer:

4) Correct Answer

Question 20:

A traditional cable TV network transmits signals ______.

Responses:

1) upstream

2) downstream

3) upstream and downstream

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 21:

The original telephone network, which is referred to as the plain old telephone system (POTS), was a (an) ________ system.

Responses:

1) digital

2) analog

3) digital as well as analog

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 22:

The traditional cable TV system used ________cable end to end.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 23:

The second generation of cable networks is called a(n) _________ network.

Responses:

1) HFC

2) HCF

3) CFH

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 24:

The HFC network uses _______ cable.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) a combination of coaxial and fiber-optic

Answer:

4) Correct Answer

Question 25:

To use a cable network for data transmission, we need two key devices: a ______  and a _________

Responses:

1) CM; CMS

2) CT; CMTS

3) CM; CMTS

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 26:

A (An) ______ signal is the corresponding optical signals of an electrical signal level STS-n.

Responses:

1) OC-n

2) TDM-n

3) FDM-n

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 27:

SONET uses _______ TDM multiplexing.

Responses:

1) asynchronous

2) synchronous

3) statistical

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 28:

A SONET system can use _________.

Responses:

1) STS multiplexers

2) regenerators

3) add/drop multiplexers

4) All of the choices are correct.

Answer:

4) Correct Answer

Question 29:

SONET sends ________ frames per second

Responses:

1) 1000

2) 2000

3) 4000

4) 8000

Answer:

4) Correct Answer

Question 30:

In SONET, __________ allow insertion and extraction of signals.

Responses:

1) regenerators

2) ADMs

3) STS multiplexer/demultiplexers

4) none of the choices are correct

Answer:

2) Correct Answer

Question 31:

In SONET, for each byte, the bits are transmitted ____________.

Responses:

1) from least significant to the most significant

2) from most significant to the least significant

3) two at a time

4) three at a time

Answer:

2) Correct Answer

Question 32:

________ is the cell relay protocol designed by the corresponding Forum and adopted by the ITU-T.

Responses:

1) SONET

2) ADM

3) ATM

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 33:

The ATM standard defines ______ layers.

Responses:

1) two

2) three

3) four

4) five

Answer:

2) Correct Answer

Question 34:

An ATM data unit is a cell composed of ______ bytes.

Responses:

1) 40

2) 50

3) 52

4) 53

Answer:

4) Correct Answer

Question 35:

In ATM, a(n) ______ is the interface between a user and an ATM switch.

Responses:

1) UNI

2) NNI

3) NUI

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 36:

In ATM, a(n) _________ is the interface between two ATM switches.

Responses:

1) UNI

2) NNI

3) NUI

4) None of the choices are correct.

Answer:

Question 37:

In ATM, the _______layer accepts transmissions from upper-layer services and maps them into cells.

Responses:

1) physical

2) ATM

3) AAL

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 38:

In ATM, the ______ layer provides routing, traffic management, switching, and multiplexing services.

Responses:

1) physical

2) ATM

3) AAL

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 39:

In SONET, a ________ is a repeater.

Responses:

1) regenerator

2) ADM

3) STS multiplexer/demultiplexer

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 40:

Which of the following statements about wired and wireless LANs is correct?

Responses:

1) Both operate in the lower two layers of the TCP/IP protocol suite.

2) Wired LANs operate in the lower two layers and wireless LANs operate in the lower three layers of the TCP/IP protocol suite.

3) Wired LANs operate in the lower three layers and wireless LANs operate in the lower two layers of the TCP/IP protocol suite.

4) Both operate in the lower three layers of the TCP/IP protocol suite.

Answer:

1) Correct Answer

Question 41:

IEEE has defined the specifications for a wireless LAN, called _______, which covers the physical and data link layers.

Responses:

1) IEEE 802.3

2) IEEE 802.5

3) IEEE 802.11

4) IEEE 802.2

Answer:

3) Correct Answer

Question 42:

The CSMA/CD algorithm does not work in wireless LAN because

Responses:

1) wireless host does not have enough power to work in s duplex mode.

2) of the hidden station problem.

3) signal fading could prevent a station at one end from hearing a collision at the other end.

4) All of the choices are correct.

Answer:

4) Correct Answer

Question 43:

In IEEE 802.11, a ___ is made of stationary or mobile wireless stations and an optional central base station, known as the access point (AP).

Responses:

1) ESS

2) BSS

3) CSS

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 44:

In IEEE 802.11, a BSS without an AP is called _________.

Responses:

1) an ad hoc architecture

2) an infrastructure network

3) either an ad hoc architecture or an infrastructure network

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 45:

In IEEE 802.11, a BSS with an AP is sometimes referred to as ____________.

Responses:

1) an ad hoc architecture

2) an infrastructure network

3) either an ad hoc architecture or an infrastructure network

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 46:

In IEEE 802.11, communication between two stations in two different BSSs usually occurs via two ________.

Responses:

1) BSSs

2) ESSs

3) APs

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 47:

In IEEE 802.11, a station with ________ mobility is either stationary (not moving) or moving only inside a BSS.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 48:

In IEEE 802.11, a station with ________mobility can move from one BSS to another, but the movement is confined inside one ESS.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 49:

In IEEE 802.11, a station with ________ mobility can move from one ESS to another.

Responses:

1) no-transition

2) BSS-transition

3) ESS-transition

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 50:

In IEEE 802.11, distributed coordination function (DCF) uses _______ as the access method.

Responses:

1) CSMA/CA

2) CSMA/CD

3) ALOHA

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 51:

In IEEE 802.11, when a frame is going from one station in a BSS to another station in the same BSS, the address flag is _____

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

1) Correct Answer

Question 52:

In IEEE 802.11, when a frame is coming from an AP and going to a station, the address flag is _______.

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

2) Correct Answer

Question 53:

In IEEE 802.11, when a frame is going from a station to an AP, the address flag is _____.

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

3) Correct Answer

Question 54:

In IEEE 802.11, when a frame is going from one AP to another AP in a wireless distribution system, the address flag is _____

Responses:

1) 00

2) 01

3) 10

4) 11

Answer:

4) Correct Answer

Question 55:

In IEEE 802.11, the access method used in the PCF sublayer is ______.

Responses:

1) contention

2) controlled

3) polling

4) None of the choices are correct.

Answer:

3) Correct Answer

Question 56:

In IEEE 802.11, the ______ is a time period used for collision avoidance.

Responses:

1) NAV

2) BSS

3) ESS

4) None of the choices are correct.

Answer:

1) Correct Answer

Question 57:

In IEEE 802.11, the addressing mechanism can include up to ______addresses.

Responses:

1) four

2) five

3) six

4) None of the choices are Correct.

Answer:

1) Correct Answer

Question 58:

The original IEEE 802.11 uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

4) Correct Answer

Question 59:

The IEEE 802.11a, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

3) Correct Answer

Question 60:

The IEEE 802.11b, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

2) Correct Answer

Question 61:

The IEEE 802.11g, uses _________.

Responses:

1) FHSS

2) DSSS

3) OFDM

4) either FHSS or DSSS

Answer:

3) Correct Answer

Question 62:

The IEEE 802.11 FHSS uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

2) Correct Answer

Question 63:

The IEEE 802.11 or IEEE 802.11b DSSS uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

3) Correct Answer

Question 64:

The IEEE 802.11a, IEEE 802.11g, or IEEE 802.11n OFDM uses ______ modulation.

Responses:

1) ASK

2) FSK

3) PSK

4) None of the choices are Correct.

Answer:

4) Correct Answer

Question 65:

Bluetooth is a _______ technology that connects devices (called gadgets) in a small area.

Responses:

1) wired LAN

2) wireless LAN

3) VLAN

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 66:

In Bluetooth, multiple ________ form a network called a _________.

Responses:

1) scatternet; piconets

2) piconets: scatternet

3) piconets: bluenet

4) bluenet; scatternet

Answer:

2) Correct Answer

Question 67:

A Bluetooth network consists of _____ primary device(s) and up to ____ secondary devices.

Responses:

1) one; five

2) five; three

3) two; six

4) one; seven

Answer:

4) Correct Answer

Question 68:

In Bluetooth, the current data rate is ____Mbps

Responses:

1) 2

2) 5

3) 11

4) None of the choices are correct.

Answer:

4) Correct Answer

Question 69:

The access method in Bluetooth is ________.

Responses:

1) FDMA

2) TDD-TDMA

3) CDMA

4) None of the choices are correct.

Answer:

2) Correct Answer

Question 70:

In Bluetooth, the _____ link is used when data integrity is more important than avoiding latency.

Responses:

1) SCO

2) ACL

3) ACO

4) SCL

Answer:

2) Correct Answer

Question 71:

Bluetooth uses ______ in the physical layer to avoid interference from other devices or other networks.

Responses:

1) DSSS

2) FHSS

3) FDMA

4) None of the choices are correct.

Answer:

2) Correct Answer