Review.   BRING SCANTRON, SAME AS THE ONE USED FOR MIDTERM

Review

| HLEN TTL – maximum hop counts, hop count to restrict to the LAN. Encapsulation Packet Frame Datagram ICMP SSID Error Messages – ICMP messages of various types and number associated with them DHCP, BOOTP, RARP Logical and physical addresses Total length, header length, data length Ping Traceroute TCP, UDP | TCP/IP layers Distance Vector RIP problems Dynamic IP DNS ARP VPN Digital Certificates Firwall – Packet filtering and Proxy BGP RIP OSPF MPLS ATM Special IP addresses: Network address, directed broadcast address, limited broadcast address, this computer, and loopback addresss. | Replay Buffer overflow Address spoofing SYN flood Phishing Denial of service Router configuration. CISCO IOS Piggybacking Distance Vector – route discovery & message intervals Split horizon EGRP NAT Assembly and Reassembly of Packets Shortest Path computation Longest prefix match Default Routes VLAN Bridge |
| --- | --- | --- |

Study

Write a one-page summary of IOT that we discussed in class.

Security Question: If Alice wants to send Bob an encrypted message using asymmetric Key how it done?  How would it differ from a symmetric key encryption?  What is digital signature?  Ho could Alice be sure that a message was sent by Bob?

A packet with 4000 bytes needs to be sent.  Fragmentation is allowed.  It first passes through a network with a MTU of 1400, later it travels through a network with a MTU of 800.  Show the fragments formed in the first network, and show the second fragment from this one being fragmented as it travels through the second network.  I am looking specifically for Header length, Total Length, fragmentation flags, and fragment offset fields.  Also show the byte numbers in each fragment.  Also, explain how reassembly takes place.

Compare and contrast physical vs. IP address schemes.  Make sure to include: (a) How these addresses are assigned, (b) Under which layer in which each layer operate, (c) How an unknown destination address is discovered (what protocols or servers are used), (d) One of these addresses can be divided into two parts, what are they and how can these parts be isolated?

Differentiate between a hub, repeater, bridge, switch and a router.  Discuss the layers each operate under, which header each is capable of inspecting, and applicable use for each.

Several methods of gaining access to the media (MAC layer responsibility) were described in class.  Explain the most common one.  Explain static allocation models and collision free protocols.

Explain ARP and RARP protocols.  Why are they used?

Explain sliding windows as related to TCP how it is used to control flow rate.  Discuss IGRP and EGRP, and differentiate between RIP and OSPF.

Discuss how NAT aids in security associated with a Firewall.

Explain how the port numbers are used by UDP and TCP and why they are used.  Also explain the difference between passive and active opens.

Alice sends a digitally signed document to Fred.  Describe the process of digital signature and how Alice can’t repudiate.  If she wants to send the signed document securely, how can that be done?

What is the difference between symmetric and asymmetric encryption?  Describe appropriate uses for both.  List some of the technologies and protocols used for both.

Summarize your practical project in less than 2 pages.

Explain failover clustering and describe how it was set up.

Problems

A packet has arrived with M bit value of 1 and fragmentation offset is zero.  Is this first, middle or last fragment?  Explain.  What if the offset was 100, what would be the first byte number (from the original whole packet) in this fragment? Explain.

An IP datagram arrived with the following in the header (HEX – I ADDED THE SPACE FOR CLARITY):

45 00 00 54 00 03 00 00 20 06 00 00 7C 4E 03 02 B4 0E 0F 02

You will be asked all sorts of questions regarding this.

Using the following diagram, create a routing table for R1, R2 and R3

Given a CIDR address, find the first usable address and netmask.

Given a classfull network address and organizational requirements, make subnets and subnet masks. Example question: A class C address was given to your company.  You have 4 departments with approximately equal number of computers and printers with a total of 100 devices requiring IP addresses.  Make a subnet and explain your reasoning.

UTRGV is given a class B IP address: 129.113.0.0.  The computer science department requires 312 IP addresses.  Create a CIDR address to hand to the computer science department.  What would be subnet mask for a classfull address?