Chapter 8

Question 1:

Routing inside an autonomous system is referred to as ________ routing.

Responses:

1) interdomain

2) intradomain

3) out-of-domain

4) None of the choices are correct

Answer:

2) Correct Answer

Question 2:

Routing between autonomous systems is referred to as ______ routing.

Responses:

1) interdomain routing

2) intradomain routing

3) out-of-domain

4) None of the choices are correct

Answer:

1) Correct Answer

Question 3:

In _______ routing, the least cost route between any two nodes is the route with the minimum distance.

Responses:

1) path vector

2) distance vector

3) link state

4) None of the choices are correct

Answer:

2) Correct Answer

Question 4:

In ________, each node maintains a vector (table) of minimum distances to every node.

Responses:

1) path vector

2) distance vector

3) link state

4) None of the choices are correct

Answer:

2) Correct Answer

Question 5:

In distance vector routing, each node periodically shares its routing table with _________ and whenever there is a change.

Responses:

1) every other node

2) its immediate neighbors

3) one neighbor

4) None of the choices are correct

Answer:

2) Correct Answer

Question 6:

The Routing Information Protocol (RIP) is an intradomain routing based on _________ routing.

Responses:

1) distance vector

2) link state

3) path vector

4) None of the choices are correct

Answer:

1) Correct Answer

Question 7:

The metric used by _______ is the hop count.

Responses:

1) OSPF

2) RIP

3) BGP

4) None of the choices are correct

Answer:

2) Correct Answer

Question 8:

In RIP, the ________ timer controls the advertising of regular update messages.

Responses:

1) garbage collection

2) expiration

3) periodic

4) None of the choices are correct

Answer:

3) Correct Answer

Question 9:

In RIP, the ________ timer is used to purge invalid routes from the table.

Responses:

1) garbage collection

2) expiration

3) periodic

4) None of the choices are correct

Answer:

1) Correct Answer

Question 10:

In RIP, the ________ timer controls the validity of the route.

Responses:

1) garbage collection

2) expiration

3) periodic

4) None of the choices are correct

Answer:

2) Correct Answer

Question 11:

RIP uses the services of _______.

Responses:

1) TCP

2) UDP

3) IP

4) None of the choices are correct

Answer:

2) Correct Answer

Question 12:

The _________ routing uses the Dijkstra algorithm to build a routing table.

Responses:

1) distance vector

2) link state

3) path vector

4) None of the choices are correct

Answer:

2) Correct Answer

Question 13:

The Open Shortest Path First (OSPF) protocol is an intradomain routing protocol based on _______ routing.

Responses:

1) distance vector

2) link state

3) path vector

4) None of the choices are correct

Answer:

2) Correct Answer

Question 14:

The _______ protocol allows the administrator to assign a cost, called the metric, to each route.

Responses:

1) OSPF

2) RIP

3) BGP

4) None of the choices are correct

Answer:

1) Correct Answer

Question 15:

In OSPF, a ________ link connects two routers without any other host or router in between.

Responses:

1) point-to-point

2) transient

3) stub

4) None of the choices are correct

Answer:

1) Correct Answer

Question 16:

In OSPF, a _______ link is a network with several routers attached to it.

Responses:

1) point-to-point

2) transient

3) stub

4) None of the choices are correct

Answer:

2) Correct Answer

Question 17:

In OSPF, a ________ link is a network connected to only one router.

Responses:

1) point-to-point

2) transient

3) stub

4) None of the choices are correct

Answer:

3) Correct Answer

Question 18:

In OSPF, a ________ defines the links of a network.

Responses:

1) network link

2) router link

3) summary link to network

4) None of the choices are correct

Answer:

1) Correct Answer

Question 19:

___________ is an interdomain routing protocol using path vector routing.

Responses:

1) BGP

2) RIP

3) OSPF

4) None of the choices are correct

Answer:

1) Correct Answer

Question 20:

A one-to-all communication between one source and all hosts on a network is classified as a _______ communication.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct

Answer:

3) Correct Answer

Question 21:

A one-to-many communication between one source and a specific group of hosts is classified as a _______ communication.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct

Answer:

2) Correct Answer

Question 22:

A one-to-one communication between one source and one destination is classified as a _______ communication.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct

Answer:

1) Correct Answer

Question 23:

In ______, the router forwards the received packet through only one of its interfaces.

Responses:

1) unicasting

2) multicasting

3) broadcasting

4) None of the choices are correct

Answer:

1) Correct Answer

Question 24:

In multicast routing, each involved router needs to construct a ________ path tree for each group.

Responses:

1) average

2) longest

3) shortest

4) None of the choices are correct

Answer:

3) Correct Answer

Question 25:

In the _______ tree approach to multicasting, each router needs to create a separate tree for each source-group.

Responses:

1) group-shared

2) source-based

3) destination-based

4) None of the choices are correct

Answer:

2) Correct Answer

Question 26:

The Multicast Open Shortest Path First (MOSPF) routing uses the _______ tree approach.

Responses:

1) source-based

2) group-shared

3) destination-based

4) None of the choices are correct

Answer:

1) Correct Answer

Question 27:

The Multicast Open Shortest Path First (MOSPF) protocol is an extension of the OSPF protocol that uses multicast routing to create source-based trees. The protocol is based on _______ routing.

Responses:

1) distance vector

2) link state

3) path vector

4) None of the choices are correct

Answer:

2) Correct Answer

Question 28:

In RPF, a router forwards only the copy that has traveled the _______ path from the source to the router.

Responses:

1) shortest

2) longest

3) average

4) None of the choices are correct

Answer:

1) Correct Answer

Question 29:

RPF eliminates the ________ in the flooding process.

Responses:

1) forwarding

2) backwarding

3) flooding

4) None of the choices are correct

Answer:

4) Correct Answer

Question 30:

RPB creates a shortest path _______ tree from the source to each destination.

Responses:

1) unicast

2) multicast

3) broadcast

4) None of the choices are correct

Answer:

3) Correct Answer

Question 31:

RPB guarantees that each destination receives _________ of the packet.

Responses:

1) only one copy

2) no copies

3) multiple copies

4) None of the choices are correct

Answer:

1) Correct Answer

Question 32:

In ________, the multicast packet must reach only those networks that have active members for that particular group.

Responses:

1) RPF

2) RPB

3) RPM

4) None of the choices are correct

Answer:

3) Correct Answer

Question 33:

_______ adds pruning and grafting to _______  create a multicast shortest path tree that supports dynamic membership changes.

Responses:

1) RPM; RPB

2) RPB; RPM

3) RPF; RPM

4) None of the choices are correct

Answer:

1) Correct Answer

Question 34:

__________ is an implementation of multicast distance vector routing. It is a source-based routing protocol, based on RIP.

Responses:

1) MOSPF

2) DVMRP

3) CBT

4) None of the choices are correct

Answer:

2) Correct Answer

Question 35:

DVMRP is a ________routing protocol, based on RIP.

Responses:

1) source-based

2) group-shared

3) destination-based

4) None of the choices are correct

Answer:

1) Correct Answer

Question 36:

Pruning and grafting are strategies used in _______.

Responses:

1) RPF

2) RPB

3) RPM

4) None of the choices are correct

Answer:

3) Correct Answer

Question 37:

PIM-DM is used when the number of routers with attached members is ______ relative to the number of routers in the internet.

Responses:

1) large

2) small

3) moderate

4) None of the choices are correct

Answer:

1) Correct Answer

Question 38:

PIM-SM is used when the number of routers with attached members is ______ relative to the number of routers in the internet.

Responses:

1) large

2) small

3) moderate

4) None of the choices are correct

Answer:

2) Correct Answer