Chapter 2

Question 1:

In a frequency-domain plot, the horizontal axis measures the ________.

Responses:

1) signal amplitude

2) frequency

3) phase

4) time

Question 2:

In a time-domain plot, the horizontal axis is a measure of ________.

Responses:

1) signal amplitude

2) frequency

3) phase

4) time

Question 3:

_______ data are continuous and take continuous values.

Responses:

1) analog

2) digital

3) analog or digital

4) None of the choices are correct.

Question 4:

_______ data have discrete states and take discrete values.

Responses:

1) analog

2) digital

3) analog or digital

4) None of the choices are correct.

Question 5:

_____ signals have an infinite number of values in a time interval.

Responses:

1) Analog

2) Digital

3) Either analog or digital

4) None of the choices are correct.

Question 6:

_______ signals can have only a limited number of values in a time interval.

Responses:

1) Analog

2) Digital

3) Either analog or digital

4) None of the choices are correct.

Question 7:

Frequency and period are ______.

Responses:

1) inverse of each other

2) proportional to each other

3) the same

4) are not related

Question 8:

________is the rate of change with respect to time.

Responses:

1) Amplitude

2) Time

3) Frequency

4) Phase

Question 9:

_______ describes the position of the waveform relative to time 0.

Responses:

1) Amplitude

2) Time

3) Frequency

4) Phase

Question 10:

A simple sine wave can be represented by one single spike in the _____ domain.

Responses:

1) amplitude

2) time

3) frequency

4) phase

Question 11:

As frequency increases, the period ________.

Responses:

1) decreases

2) increases

3) remains the same

4) None of the choices are correct.

Question 12:

________ is a type of transmission impairment in which the signal loses strength due to the resistance of the transmission medium.

Responses:

1) Attenuation

2) Distortion

3) Noise

4) Decibel

Question 13:

________ is a type of transmission impairment in which the signal loses strength due to the different propagation speeds of each frequency that makes up the signal.

Responses:

1) Attenuation

2) Distortion

3) Noise

4) Decibel

Question 14:

________ is a type of transmission impairment in which an outside source such as crosstalk corrupts a signal.

Responses:

1) Attenuation

2) Distortion

3) Noise

4) Decibel

Question 15:

When propagation speed is multiplied by propagation time, we get the ________.

Responses:

1) throughput

2) wavelength of the signal

3) distortion factor

4) distance a signal or bit has traveled

Question 16:

Baseband transmission of a digital signal is possible only if we have a ____ channel.

Responses:

1) lowpass

2) bandpass

3) low rate

4) high rate

Question 17:

If the available channel is a ____ channel, we cannot send a digital signal directly to the channel.

Responses:

1) lowpass

2) bandpass

3) low rate

4) high rate

Question 18:

For a ______ channel, the Nyquist bit rate formula defines the theoretical maximum bit rate.

Responses:

1) noisy

2) noiseless

3) bandpass

4) lowpass

Question 19:

For a ______ channel, we need to use the Shannon capacity to find the maximum bit rate.

Responses:

1) noisy

2) noiseless

3) bandpass

4) lowpass

Question 20:

_________ can impair a signal.

Responses:

1) Attenuation

2) Distortion

3) Noise

4) All of the choices are correct.

Question 21:

The _________ product defines the number of bits that can fill the link.

Responses:

1) bandwidth-period

2) frequency-amplitude

3) bandwidth-delay

4) delay-amplitude

Question 22:

Polar and bipolar encoding are types of _______ coding.

Responses:

1) line

2) block

3) scrambling

4) None of the choices are correct.

Question 23:

___________ conversion involves three techniques: line coding, block coding, and scrambling.

Responses:

1) Analog-to-digital

2) Digital-to-analog

3) Analog-to-analog

4) Digital-to-digital

Question 24:

In ______ schemes, the voltage level oscillates between a positive and a negative value although it may remain at zero level between the two values.

Responses:

1) polar

2) bipolar

3) nonpolar

4) None of the choices are correct.

Question 25:

In _____, the level of the voltage determines the value of the bit.

Responses:

1) NRZ-I

2) NRZ-L

3) NRZ-I or NRZ-L

4) None of the choices are correct.

Question 26:

In ______, the change or lack of change in the level of the voltage determines the value of the bit.

Responses:

1) NRZ-I

2) NRZ-L

3) NRZ-I or NRZ-L

4) None of the choices are correct.

Question 27:

The idea of RZ and the idea of NRZ-L are combined into the ________ scheme.

Responses:

1) Manchester

2) differential Manchester

3) Manchester or differential Manchester

4) None of the choices are correct.

Question 28:

The idea of RZ and the idea of NRZ-I are combined into the ________ scheme.

Responses:

1) Manchester

2) differential Manchester

3) Manchester or differential Manchester

4) None of the choices are correct.

Question 29:

In Manchester and differential Manchester encoding, the transition at the middle of the bit is used for __________.

Responses:

1) bit transfer

2) baud transfer

3) synchronization

4) None of the choices are correct.

Question 30:

In _______encoding, we use three levels: positive, zero, and negative.

Responses:

1) polar

2) bipolar

3) nonpolar

4) None of the choices are correct.

Question 31:

The _____ scheme uses data patterns of size 2 and encodes the 2-bit patterns as one signal element belonging to a four-level signal.

Responses:

1) 4B5B

2) 2B1Q

3) B8ZS

4) None of the choices are correct.

Question 32:

_______ encoding has a transition at the middle of each bit.

Responses:

1) RZ

2) Manchester

3) Differential Manchester

4) All the choices are correct.

Question 33:

_______ encoding has a transition at the beginning of each 0 bit.

Responses:

1) RZ

2) Manchester

3) Differential Manchester

4) All the choices are correct.

Question 34:

Which of the following encoding methods does not provide for synchronization?

Responses:

1) NRZ-L

2) RZ

3) NRZ-I

4) Manchester

Question 35:

Which encoding method uses alternating positive and negative values for 1s?

Responses:

1) NRZ-I

2) RZ

3) Manchester

4) AMI

Question 36:

Block coding can help in _______ and _________ at the receiver.

Responses:

1) synchronization and error detection

2) synchronization and attenuation

3) error detection and attenuation

4) error detection and distortion

Question 37:

________ is the process of converting digital data to a digital signal.

Responses:

1) Block coding

2) Line coding

3) Scrambling

4) All of the choices are correct.

Question 38:

_______ provides redundancy to ensure synchronization and inherent error detection.

Responses:

1) Block coding

2) Line coding

3) Line coding or block coding

4) None of the choices are correct

Question 39:

________ is normally referred to as mB/nB coding; it replaces each m-bit group with an n-bit group.

Responses:

1) Block coding

2) Line coding

3) Scrambling

4) None of the choices are correct.

Question 40:

________ provides synchronization without increasing the number of bits.

Responses:

1) Scrambling

2) Line coding

3) Block coding

4) None of the choices are correct.

Question 41:

Two common scrambling techniques are ________.

Responses:

1) NRZ and RZ

2) AMI and NRZ

3) B8ZS and HDB3

4) Manchester and differential Manchester

Question 42:

PCM is an example of _______ conversion.

Responses:

1) digital-to-digital

2) digital-to-analog

3) analog-to-analog

4) analog-to-digital

Question 43:

The most common technique to change an analog signal to digital data is called __________.

Responses:

1) PAL

2) PCM

3) sampling

4) None of the choices are correct.

Question 44:

The first step in PCM is ________.

Responses:

1) quantization

2) modulation

3) sampling

4) None of the choices are correct.

Question 45:

______ finds the value of the signal amplitude for each sample; ____ finds the change from the previous sample.

Responses:

1) DM; PCM

2) PCM; DM

3) DM; CM

4) None of the choices are correct.

Question 46:

ASK, PSK, FSK, and QAM are examples of ________ conversion.

Responses:

1) digital-to-digital

2) digital-to-analog

3) analog-to-analog

4) analog-to-digital

Answer:

2) Correct Answer

Question 47:

AM, FM, and PM are examples of ________ conversion.

Responses:

1) digital to digital

2) digital to analog

3) analog to analog

4) analog to digital

Question 48:

In QAM, both ________ of a carrier frequency are varied.

Responses:

1) frequency and amplitude

2) phase and frequency

3) amplitude and phase

4) none of the choices are correct

Question 49:

In ________, the amplitude of the carrier signal is varied to create signal elements. Both frequency and phase remain constant.

Responses:

1) ASK

2) PSK

3) FSK

4) QAM

Question 50:

In _________, the frequency of the carrier signal is varied to represent data. Both peak amplitude and phase remain constant.

Responses:

1) ASK

2) PSK

3) FSK

4) QAM

Question 51:

In ________, the phase of the carrier is varied to represent two or more different signal elements. Both peak amplitude and frequency remain constant.

Responses:

1) ASK

2) PSK

3) FSK

4) QAM

Question 52:

Quadrature amplitude modulation (QAM) is a combination of ___________.

Responses:

1) ASK and FSK

2) ASK and PSK

3) PSK and FSK

4) None of the choices are correct.

Question 53:

________ uses two carriers, one in-phase and the other quadrature.

Responses:

1) ASK

2) PSK

3) FSK

4) QAM

Question 54:

How many carrier frequencies are used in BASK?

Responses:

1) 1

2) 2

3) 3

4) None of the choices are correct.

Question 55:

How many carrier frequencies are used in BFSK?

Responses:

1) 1

2) 2

3) 3

4) None of the choices are correct.

Question 56:

How many carrier frequencies are used in BPSK?

Responses:

1) 1

2) 2

3) 3

4) None of the choices are correct.

Question 57:

Which of the following is not an analog-to-analog conversion?

Responses:

1) AM

2) PM

3) FM

4) QAM

Question 58:

In _____ transmission, the carrier signal is modulated so that its amplitude varies with the changing amplitudes of the modulating signal.

Responses:

1) AM

2) PM

3) FM

4) None of the choices are correct.

Question 59:

In the analog transmission of the digital signal, the baud rate is ___________ the bit rate.

Responses:

1) always less than

2) less than or equal to

3) always greater than

4) is greater than or equal to.

Question 60:

In ASK, the bandwidth is ________ .

Responses:

1) less than the signal rate

2) equal or greater than signal rate

3) always equal to signal rate

4) five times the signal rate.

Question 61:

With the same signal rate, the bandwidth of FSK is normally ___________  the bandwidth for ASK.

Responses:

1) greater than

2) less than

3) equal to

4) None of the choices are correct.

Question 62:

The bandwidth of an AM signal is ______________ the bandwidth of the original analog signal.

Responses:

1) equal to

2) two times

3) three times

4) None of the choices are correct.

Question 63:

In AM radio, the allocated bandwidth for each station is ___________  kHz.

Responses:

1) 15

2) 20

3) 10

4) None of the choices are correct.

Question 64:

In FM radio, the allocated bandwidth for each station is ___________  kHz.

Responses:

1) 100

2) 200

3) 300

4) None of the choices are correct.

Question 65:

Which multiplexing technique is used for analog signals?

Responses:

1) FDM

2) TDM

3) WDM

4) PDM

Question 66:

Which multiplexing technique is used for digital signals?

Responses:

1) FDM

2) TDM

3) WDM

4) PDM

Question 67:

Which multiplexing technique shifts each signal to a different carrier frequency?

Responses:

1) FDM

2) TDM

3) WDM

4) PDM

Question 68:

Which multiplexing technique involves signals composed of light beams?

Responses:

1) FDM

2) TDM

3) WDM

4) PDM

Question 69:

________ is the set of techniques that allows the simultaneous transmission of multiple signals across a single data link.

Responses:

1) Demodulating

2) Multiplexing

3) Compressing

4) None of the choices are correct

Question 70:

____ is designed to use the high bandwidth capability of fiber-optic cable.

Responses:

1) FDM

2) TDM

3) WDM

4) None of the choices are correct.

Question 71:

______ is an analog multiplexing technique to combine optical signals.

Responses:

1) FDM

2) TDM

3) WDM

4) None of the choices are correct.

Question 72:

_____ is a digital process that allows several connections to share the high bandwidth of a link.

Responses:

1) FDM

2) TDM

3) WDM

4) None of the choices are correct.

Question 73:

We can divide ____ into two different schemes: synchronous or statistical.

Responses:

1) FDM

2) TDM

3) WDM

4) None of the choices are correct.

Question 74:

In ________ TDM, each input connection has an allotment in the output even if it is not sending data.

Responses:

1) synchronous

2) statistical

3) isochronous

4) None of the choices are correct.

Question 75:

In ________ TDM, slots are dynamically allocated to improve bandwidth efficiency.

Responses:

1) synchronous

2) statistical

3) isochronous

4) None of the choices are correct.

Question 76:

The _______ technique uses M different carrier frequencies that are modulated by the source signal. At one moment, the sign modulates one carrier frequency; at the next moment, the signal modulates another carrier frequency.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 77:

The ______ technique expands the bandwidth of a signal by replacing each data bit with n bits using a spreading code.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 78:

Groups, super groups, master groups, and jumbo groups are terms used in ______________.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 79:

Multilevel multiplexing is a strategy used in ___________.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 80:

Multi-slot allocation is a strategy used in ___________.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 81:

Pulse stuffing  is a strategy used in ___________.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 82:

Frame synchronization is a strategy used in ___________.

Responses:

1) FDM

2) DSSS

3) FHSS

4) TDM

Question 83:

A T-1 line uses   ___________ frames.

Responses:

1) 6000

2) 8000

3) 10000

4) 12000

Question 84:

We need addressing mechanism in  ___________ TDM.

Responses:

1) synchronous

2) statistical

3) both synchronous and statistical

4)

Question 85:

Transmission media are usually categorized as _______.

Responses:

1) fixed or unfixed

2) guided or unguided

3) determinate or indeterminate

4) metallic or nonmetallic

Question 86:

Transmission media lie below the _______ layer.

Responses:

1) physical

2) network

3) transport

4) application

Question 87:

_______ cable consists of an inner copper core and a second conducting outer sheath.

Responses:

1) Twisted pair

2) Coaxial

3) Fiber-optic

4) Shielded twisted air

Question 88:

In fiber optics, the signal is _______ waves.

Responses:

1) light

2) radio

3) infrared

4) very low frequency

Question 89:

Which of the following is not a guided medium?

Responses:

1) twisted-pair cable

2) coaxial cable

3) fiber-optic cable

4) atmosphere

Question 90:

Which of the following is not an unguided medium?

Responses:

1) twisted-pair cable

2) coaxial cable

3) fiber-optic cable

4) None of the choices are correct

Question 91:

Twisting in a twisted-pair help reduce the __________.

Responses:

1) length

2) cost

3) noise

4) None of the choices are correct

Question 92:

Noise in a coaxial cable is reduced by ___________________.

Responses:

1) twisting the cable

2) the outer conductor

3) the inner conductor

4) None of the choices are correct

Question 93:

Noise in a coaxial cable is reduced by ___________________.

Responses:

1) twisting the cable

2) the outer conductor

3) the inner conductor

4) None of the choices are correct

Question 94:

UTP and STP are different implementations of ___________________ cable.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) None of the choices are correct

Question 95:

RJ-45 is a type of connectors used in   _________ cabling.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) None of the choices are correct

Question 96:

RG rating is used in _________ cable.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) None of the choices are correct

Question 97:

SC and TP are two types of connectors used in   _________ cabling.

Responses:

1) twisted pair

2) coaxial

3) fiber-optic

4) None of the choices are correct

Question 98:

The infra-red wave has frequencies ________ microwave.

Responses:

1) below

2) above

3) the same as

4) None of the choices are correct

Question 99:

Circuit switching can be divided into   ________ categories.

Responses:

1) two

2) three

3) four

4) None of the choices are correct

Question 100:

Packet switching can be divided into   ________ categories.

Responses:

1) two

2) three

3) four

4) None of the choices are correct

Question 101:

Circuit switching is normally used in ___________ layer.

Responses:

1) physical

2) data link

3) network

4) application

Question 102:

Packet switching is normally used in ___________ layers.

Responses:

1) physical and data link

2) data link and network

3) network and transport

4) transport and application

Question 103:

Message switching is normally used in ___________ layer.

Responses:

1) physical

2) data link

3) network

4) application

Question 104:

In a circuit-switching network, we have   ___________ phase(s).

Responses:

1) one

2) two

3) three

4) None of the choices are correct

Question 105:

In a datagram network, we have   ___________ phase(s).

Responses:

1) one

2) two

3) three

4) None of the choices are correct

Question 106:

In a virtual-switch network, we have   ___________ phase(s).

Responses:

1) one

2) two

3) three

4) None of the choices are correct

Question 107:

In a ___________ network, each packet is treated independently from all other packets.

Responses:

1) circuit-switched

2) virtual circuit

3) datagram

4) None of the choices are correct

Question 108:

In a datagram network, the routing table is based on the   ___________ in the packet.

Responses:

1) flow label

2) destination address

3) VCI

4) None of the choices are correct

Question 109:

In a virtual-circuit network, the routing table is based on the   ___________ in the packet.

Responses:

1) flow label

2) destination address

3) VCI

4) None of the choices are correct

Question 110:

In a datagram network, the destination address ________________________.

Responses:

1) remains the same from the source to the destination

2) changes at each switch

3) changes at the destination

4) None of the choices are correct

Question 111:

In a datagram network, we need ______________ phase(s) .

Responses:

1) tear-down

2) setup

3) setup and tear-down

4) None of the choices are correct

Question 112:

In a virtual-circuit network, we need ______________ phase(s) .

Responses:

1) tear-down

2) setup

3) setup and tear-down

4) None of the choices are correct

Question 113:

In a __________network, all packets in a message follow the same path.

Responses:

1) datagram

2) virtual circuit

3) circuit-switched

4) None of the choices are correct

Question 114:

In a __________network, each packet in a message may follow a different path.

Responses:

1) datagram

2) virtual circuit

3) circuit-switched

4) None of the choices are correct