Chapter 6

Question 1:

A VLAN as a local area network configured by_____________.

Responses:

1) software

2) physical wiring

3) software or physical wiring

4) None of the choices are correct.

Question 2:

A repeater is a connecting device that operates in the _______ layer(s).

Responses:

1) physical

2) physical and data link

3) data link and network

4) physical, data link and network

Question 3:

A link-layer switch is a connecting device that operates in the _______ layer(s).

Responses:

1) physical

2) physical and data link

3) data link and network

4) physical, data link and network

Question 4:

A router is a connecting device that that operates in the _______ layer(s).

Responses:

1) physical

2) physical and data link

3) data link and network

4) physical, data link and network

Question 5:

A _________ has no filtering capability.

Responses:

1) repeater

2) link-layer switch

3) router

4) None of the choices are correct.

Question 6:

A three-layer switch is a __________.

Responses:

1) repeater

2) link-layer switch

3) router

4) None of the choices are correct.

Question 7:

A router filters packets based on the __________ destination addresses.

Responses:

1) datalink

2) network-layer

3) transport-layer

4) None of the choices are correct.

Question 8:

A link-layer switch filters packets based on the __________ destination addresses.

Responses:

1) datalink

2) network-layer

3) transport-layer

4) None of the choices are correct.

Question 9:

Looping problems in a set of switches can be eliminated using ___________ algorithm.

Responses:

1) routing

2) spanning tree

3) forwarding

4) None of the choices are correct.

Question 10:

Membership in a VLAN can be based on ________.

Responses:

1) interface numbers

2) MAC addresses

3) IP addresses

4) All of the choices are correct.